## Surface Wipe Sampling for Arsenic

## 1. Purpose

This describes the collection of surface wipe samples for arsenic to assess surface contamination of Hot gas Pass components

## 2. Scope

This procedure applies to wipe sampling of defined surface areas for arsenic. The method is intended for total elemental arsenic and is not species-specific (which is acceptable for Hot gas pass parts)

## 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Field Sampler | Collect samples according to this SOP, maintain sample integrity, and prevent contamination. |
| Project Lead / IH | Define sampling locations, sample numbers, QA/QC requirements, and data interpretation approach. |
| Laboratory | Perform sample digestion and analysis using the approved analytical method. |

## 4. Safety and Precautions

| Hazard / Precaution | Requirement |
| --- | --- |
| Gloves | Wear clean powder-free disposable gloves. Change gloves between samples. |
| Contamination control | Avoid touching sample surfaces, wipes, or containers with bare hands. |
| Sample integrity | Keep containers sealed after collection. |
| Chemical note | Analytical digestion may involve strong acids; laboratory staff must follow applicable chemical safety procedures. |

## 5. Materials and Equipment

| Item | Requirement |
| --- | --- |
| Wipes | Pre-packaged moist disposable wipe suitable for metal wipe sampling (e.g. Ghost wipes) |
| Gloves | Powder-free disposable gloves |
| Template | Template of known area, or tape and measuring device if template cannot be used |
| Containers | Clean hard-walled sample containers |
| Labels / COC | Sample labels and chain-of-custody forms |
| Decontamination supplies | Materials to clean template/tools between samples |

## 6. Sampling Procedure

| Step | Procedure |
| --- | --- |
| 0 | Use Appendix I: Arsenic Wipe Sampling Field Form to record sampling |
| 1 | Put on clean powder-free gloves. |
| 2 | Place and secure the template over the sampling area. If a template cannot be used, measure and delineate the area with masking tape. 10cmx10cm = 100cm2 (10cm = 3.937 in or 315/16 in ). |
| 3 | Remove one wipe from the package and unfold it. |
| 4 | Wipe the surface with firm pressure using overlapping “S” strokes in a horizontal direction. |
| 5 | Fold the wipe inward and wipe the same area using vertical “S” strokes. |
| 6 | Fold again to expose a clean side and repeat the horizontal wipe. |
| 7 | Fold the wipe with the exposed side inward and place it into a clean hard-walled sample container. |
| 8 | Seal, label, and document the sample immediately. |
| 9 | Clean or replace the template before the next sample. (or use new masking tape) |
| 10 | Remove and discard gloves. Use clean gloves for each new sample. |

## 7. Field Blanks

| Requirement | Specification |
| --- | --- |
| Frequency | Approximately 10% of samples, minimum of 1 per batch |
| Collection timing | Middle of sampling sequence |
| Procedure | Remove an unused wipe from the package and place it directly into a sample container without contacting a surface |

## 8. Sample Handling and Transport

| Requirement | Specification |
| --- | --- |
| Container closure | Seal immediately after collection |
| Labelling | Assign unique sample ID and record location, area, date, and sampler |
| Storage | Store as routine environmental samples |
| Shipment | Maintain chain of custody during transfer and shipment |

## 9. Quality Assurance / Quality Control

| QA/QC Element | Requirement |
| --- | --- |
| Sampling consistency | Use the same wipe technique and area size for comparable samples (10cm x 10cm) or |
| Field blanks | Include field blanks as specified above |

## 10. References

| Reference | Description |
| --- | --- |
| NIOSH Method 9102 mod SW846 3050B/6020A/NIOSH9102 ICPMS LDHB | Elements on Wipes |

## Appendix I:

## Arsenic Wipe Sampling Field Form

### Project Information

| Field | Entry |
| --- | --- |
| Project Name |  |
| Site / Building |  |
| Department / Area |  |
| Date |  |
| Sampler(s) |  |
| Sample Set ID |  |
| Weather / Field Conditions |  |

### Sampling Plan

| Field | Entry |
| --- | --- |
| Analyte | Arsenic |
| Sample Type | Surface wipe sample |
| Purpose |  |
| Sampling Area (cm²) |  |
| Template Used? (Yes/No) |  |
| If No, Area Delineation Method |  |
| Wipe Type / Manufacturer |  |
| Sample Container Type |  |

### Sample Log

| Sample ID | Location / Area | Surface Type | Area Sampled (cm²) | Sample / Blank | Time | Notes |
| --- | --- | --- | --- | --- | --- | --- |

### Sampling Method Details

| Field | Entry |
| --- | --- |
| Gloves Worn? (Yes/No) |  |
| Gloves Changed Between Samples? (Yes/No) |  |
| Wipe Pattern Used (Circle One) | Horizontal S-strokes / Vertical S-strokes / Horizontal S-strokes |
| Number of Wipes per Sample |  |
| Field Blanks Collected? (Yes/No) If No why? |  |
| Number of Field Blanks (> 10%) |  |
| Field Blank Distribution | Beginning / Middle / End |
| Duplicate Samples Collected? (Yes/No) |  |
| Deviations from Procedure (What & Why) |  |

### Surface Observations

| Field | Entry |
| --- | --- |
| Visible Dust / Residue |  |
| Surface Condition |  |
| Surface Material |  |
| Access / Obstructions |  |
| Other Observations |  |

### Sample Handling / Chain of Custody

| Field | Entry |
| --- | --- |
| Containers Sealed After Collection? (Yes/No) |  |
| Labels Completed? (Yes/No) |  |
| Chain of Custody Completed? (Yes/No) |  |
| Samples Stored / Shipped as Routine Samples? (Yes/No) |  |
| Shipment Date |  |
| Released By |  |
| Received By |  |

### Additional Notes

| Field | Entry |
| --- | --- |
| Comments |  |
| Corrective Actions / Follow-Up Needed |  |
| Laboratory Method Requested |  |
| Analyst / Reviewer |  |